# Checklist Prior to Departure

  - Install STW Paddlewheel in Transducer

  - Water tanks isolated

  - Below decks

    - Galley

      - Unlock Stove Gimbals

      - All items on counters stowed or fastened

      - Pressure flush sink drains.  Use Drano if necessary.

      - Remove items beneath oven that can trip gimbal lock

    - Nav Curtain Drawn

  - Jacklines installed

  - Dinghy on deck

  - Preventer lines reeved and ready.

  - Life jackets

    - Make Available

    - Check batteries on strobe lights

  - Check all equipment tie-downs

  - Window covers removed, Canopy Off, Winch Covers off

  - Steering wheel mounted

  - Propane sniffer test

  - Stopper in Diesel Heater Exhaust Outlet

  - Sink Drains closed

  - Washdown Pump Through-Hull Valve Closed

  - Hatches and portlights closed

  - Disconnect power lines and water hose if at dock

  - Switch AC shore power to 110VAC

  - Lock down passageway access door

  - Tighten swivel nuts to lock down computer monitor

  - Cockpit rope light removed

  - Lee cloths in place.

  - Navigation

    - Routes

      - Plotted and loaded.

      - Paper charts available.

    - Instruments ON

      - VHF radio

      - SSB test

      - NAV Electronics

      - Autopilot

      - RADAR

      - SONAR

      - Zero out Trip Odometer

      - Enable Heading Sensor

  - Also check items on Daily Checklist (over).

# Daily Checklist at Sea

  - Emergency Bilge pump test

  - Day Tank fuel level

  - Check Rigging

    - Standing Rigging

      - Tension

      - Cracks

    - Running Rigging

      - Check for chafe

  - BOWT engine check

  - Bilge check: oil, coolant leaks, level, check bilge cycle meter

  - Meet Net Schedule if participating.

# Checklist after Arrival

  - Winch Covers On

  - Sail Cover On

  - Rinse Winches and cover

  - Rinse and Salt-X line clutches on deck, in cockpit, on mast.

  - Remove STW Paddlewheel

  - Turn off RADAR, A/P, NAV (unless anchor alarm is set)

  - Dodger Windows

    - Rinse with freshwater

    - Polish if necessary

    - Window covers on

  - If Docked

    - Switch AC Shore Power to 230VAC

    - connect power lines

    - wash deck

    - flush watermaker

  - If Moored or at Anchor

    - Anchor Snubber on

    - Anchor Keeper on

    - Chafe gear

    - Windlass ON

    - Set Anchor Drag Alarm if required

# Checklist if at Anchor or Moored

  - Anchor Snubber on or Chafe Gear if moored

  - Anchor Keeper on

  - Windlass ON

  - Set Anchor Drag Alarm if required

  - Check anchor gear and snubber or mooring lead each day

  - BOWT engine check once/week if charging

Cycle the disconnect switch in cabinet over galley sink after turning on NAV electronics.

BOWT = Belts, Oil, Water, Transmission