We hauled out on April 19 planning a two week haulout.  We planned to take our bottom down to gelcoat, apply a few barrier coats, three coats of bottom paint, and splash.  Instead we ended up being in the yard for 3 1/2 months doing the haulout from hell!

The short version is that we discovered a complete layer of water between the mat and roving.  How did it get there?  We will never know for sure, but we have some ideas.  Obviously we had a cold joint there or there would not have been a potential space for the water to collect.  And we believe that the water likely entered through the poor seam between the hull halves - the seams that have been discussed a bit lately.

The cure?  We removed bottom paint, peeled gelcoat, and then ground off about 1/4" of mat.  The grinding was an immense task that went on and on.

When it looked as though we had ground down to roving, we would find individual mat fibers that would have small amounts of water behind them when we ground them off.  So we went over the boat with a fine-toothed comb chasing down individual fibers.  All in all, about two months of two of us grinding.

Although some sources say that you do not need to reglass if you remove only mat, we chose to.  We laid up knytex (sp?) and two layers of 10 oz. cloth with West System epoxy.  Then fairing, fairing, fairing.  Barrier coat, bottom paint, splash.

In addition, we rebuilt our rudder and skeg (we were appalled at the quality of workmanship in the skeg), and we did quite a bit of work on the keel.  A bit of reshaping (she's faster now!) and even a strip of kevlar along the leading edge for those things that go bump in the night.

So we didn't splash until August, and we had things to do before we could leave, so we postponed again because neither of us wanted to have to speed through the south pacific to get to New Zealand before storm season.  So we will stay here till around the end of February.

Anyways, that is the short version of our haulout, and I will try to get it written up sometime soon so that others can read it and have bad dreams about hidden cold joints.